.

1. Logical Address to Physical Address Using Base and Limit Registers

Aim

To design and implement a C program that simulates the translation of a logical address into a physical address using base and limit registers.

Concept

- Logical address: Address generated by the CPU.

- Base register: Starting physical address of the process.

- Limit register: Maximum valid logical address/range.

- If the logical address is within the limit, then:

Physical Address = Base Address + Logical Address

Otherwise, the address is invalid.

Algorithm

- Start.

- Read the base address.

- Read the limit value.

- Read the logical address.

- Check whether logical_address < limit.

- If valid, calculate:
physical_address = base + logical_address

- Otherwise, display "Invalid Logical Address".

- Stop.

C Program

#include <stdio.h>

int main()

{

int base, limit, logical, physical;

printf("Enter Base Register: ");

scanf("%d", &base);

printf("Enter Limit Register: ");

scanf("%d", &limit);

printf("Enter Logical Address: ");

scanf("%d", &logical);

if (logical >= 0 && logical < limit)

{

physical = base + logical;

printf("\nAddress Translation Successful\n");

printf("Logical Address  : %d\n", logical);

printf("Base Register    : %d\n", base);

printf("Limit Register   : %d\n", limit);

printf("Physical Address : %d\n", physical);

}

else

{

printf("\nInvalid Logical Address!\n");

printf("Address is outside the allowed limit.\n");

}

return 0;

}

Sample Output – Valid Address

Enter Base Register: 1000

Enter Limit Register: 500

Enter Logical Address: 250

Address Translation Successful

Logical Address  : 250

Base Register    : 1000

Limit Register   : 500

Physical Address : 1250

Sample Output – Invalid Address

Enter Base Register: 1000

Enter Limit Register: 500

Enter Logical Address: 600

Invalid Logical Address!

Address is outside the allowed limit.

Result

Thus, the program successfully simulates logical-to-physical address translation using base and limit registers and identifies invalid logical addresses.

2. Paging Memory Management Scheme

Aim

To implement a C program that demonstrates the paging memory management scheme by dividing memory into fixed-size pages and frames and translating a logical address into a physical address using a page table.

Concept

In paging:

- Logical memory is divided into pages.

- Physical memory is divided into frames.

- Page size and frame size are equal.

- A page table maps each page to a frame.

The physical address is calculated as:

Physical Address = Frame Number × Page Size + Offset

Algorithm

- Start.

- Read the number of pages.

- Read the page size.

- Read the frame number for each page.

- Read the page number and offset.

- Check whether the page number is valid.

- Check whether the offset is within the page size.

- Obtain the frame number from the page table.

- Calculate physical address.

- Display the result.

- Stop.

C Program

#include <stdio.h>

int main()

{

int pages, pageSize;

int pageTable[100];

int page, offset, frame;

int physicalAddress;

int i;

printf("Enter number of pages: ");

scanf("%d", &pages);

printf("Enter page size: ");

scanf("%d", &pageSize);

printf("\nEnter frame number for each page:\n");

for (i = 0; i < pages; i++)

{

printf("Page %d -> Frame: ", i);

scanf("%d", &pageTable[i]);

}

printf("\nEnter page number: ");

scanf("%d", &page);

printf("Enter offset: ");

scanf("%d", &offset);

if (page < 0 || page >= pages)

{

printf("\nInvalid Page Number!\n");

}

else if (offset < 0 || offset >= pageSize)

{

printf("\nInvalid Offset!\n");

}

else

{

frame = pageTable[page];

physicalAddress = frame * pageSize + offset;

printf("\nPaging Address Translation\n");

printf("Page Number     : %d\n", page);

printf("Offset          : %d\n", offset);

printf("Frame Number    : %d\n", frame);

printf("Physical Address: %d\n", physicalAddress);

}

return 0;

}

Sample Output

Enter number of pages: 4

Enter page size: 100

Enter frame number for each page:

Page 0 -> Frame: 5

Page 1 -> Frame: 2

Page 2 -> Frame: 8

Page 3 -> Frame: 1

Enter page number: 2

Enter offset: 25

Paging Address Translation

Page Number     : 2

Offset          : 25

Frame Number    : 8

Physical Address: 825

Calculation

Physical Address

= Frame Number × Page Size + Offset

= 8 × 100 + 25

= 825

Result

Thus, the program successfully demonstrates paging memory management and logical-to-physical address translation using a page table.

3. Simulation of a Page Table

Aim

To develop a C program to simulate a page table that accepts page table entries and logical addresses and maps page numbers to corresponding frame numbers.

Concept

A page table contains the mapping:

Page Number → Frame Number

For example:

Page 0 → Frame 3

Page 1 → Frame 7

Page 2 → Frame 1

Page 3 → Frame 5

For a logical address:

Logical Address = Page Number + Offset

The corresponding physical address is:

Physical Address = Frame Number × Page Size + Offset

Algorithm

- Start.

- Read the number of pages.

- Read page size.

- Enter the frame number corresponding to each page.

- Display the page table.

- Read the logical page number.

- Read the offset.

- Find the corresponding frame number.

- Calculate physical address.

- Display page-to-frame mapping.

- Stop.

C Program

#include <stdio.h>

int main()

{

int pageTable[100];

int n, pageSize;

int page, offset;

int frame, physicalAddress;

int i;

printf("Enter number of pages: ");

scanf("%d", &n);

printf("Enter page size: ");

scanf("%d", &pageSize);

printf("\nEnter Page Table Entries:\n");

for (i = 0; i < n; i++)

{

printf("Page %d -> Frame: ", i);

scanf("%d", &pageTable[i]);

}

printf("\n----- PAGE TABLE -----\n");

printf("Page Number\tFrame Number\n");

for (i = 0; i < n; i++)

{

printf("%d\t\t%d\n", i, pageTable[i]);

}

printf("\nEnter logical page number: ");

scanf("%d", &page);

printf("Enter offset: ");

scanf("%d", &offset);

if (page < 0 || page >= n)

{

printf("\nInvalid Page Number!\n");

}

else if (offset < 0 || offset >= pageSize)

{

printf("\nInvalid Offset!\n");

}

else

{

frame = pageTable[page];

physicalAddress = frame * pageSize + offset;

printf("\nPage Table Translation\n");

printf("----------------------\n");

printf("Logical Page Number : %d\n", page);

printf("Mapped Frame Number : %d\n", frame);

printf("Offset              : %d\n", offset);

printf("Physical Address    : %d\n", physicalAddress);

}

return 0;

}

Sample Output

Enter number of pages: 4

Enter page size: 100

Enter Page Table Entries:

Page 0 -> Frame: 3

Page 1 -> Frame: 7

Page 2 -> Frame: 1

Page 3 -> Frame: 5

PAGE TABLE

Page Number    Frame Number

0              3

1              7

2              1

3              5

Enter logical page number: 1

Enter offset: 20

Page Table Translation

Logical Page Number : 1

Mapped Frame Number : 7

Offset              : 20

Physical Address    : 720

Result

Thus, the program successfully simulates a page table and demonstrates how logical page numbers are translated into physical frame numbers.

4. Translation Lookaside Buffer (TLB)

Aim

To implement a C program that simulates the working of a Translation Lookaside Buffer (TLB) and demonstrates its effect on memory access time.

Concept

A TLB is a small, fast cache that stores recently used page-to-frame mappings.

Without TLB:

CPU → Page Table → Main Memory

This generally requires two memory accesses:

1. Access page table

2. Access actual memory

With TLB hit:

CPU → TLB → Main Memory

Only one main-memory access is needed after the TLB lookup.

Formula

If:

- TLB lookup time = t

- Memory access time = m

- TLB hit ratio = h

Then:

With TLB:

Effective Access Time

= h(t + m) + (1-h)(t + 2m)

Without TLB:

Access Time = 2m

Algorithm

- Start.

- Read TLB lookup time.

- Read memory access time.

- Read TLB hit ratio.

- Calculate TLB hit time.

- Calculate TLB miss time.

- Calculate effective access time.

- Calculate access time without TLB.

- Display both values.

- Stop.

C Program

#include <stdio.h>

int main()

{

float tlbTime, memoryTime;

float hitRatio;

float hitTime, missTime;

float effectiveTime;

float withoutTLB;

printf("Enter TLB lookup time (ns): ");

scanf("%f", &tlbTime);

printf("Enter Memory access time (ns): ");

scanf("%f", &memoryTime);

printf("Enter TLB hit ratio (0 to 1): ");

scanf("%f", &hitRatio);

if (hitRatio < 0 || hitRatio > 1)

{

printf("Invalid hit ratio!\n");

return 0;

}

hitTime = tlbTime + memoryTime;

missTime = tlbTime + 2 * memoryTime;

effectiveTime = hitRatio * hitTime

+ (1 - hitRatio) * missTime;

withoutTLB = 2 * memoryTime;

printf("\n----- TLB PERFORMANCE -----\n");

printf("TLB Hit Time           : %.2f ns\n", hitTime);

printf("TLB Miss Time          : %.2f ns\n", missTime);

printf("Effective Access Time : %.2f ns\n", effectiveTime);

printf("Without TLB            : %.2f ns\n", withoutTLB);

if (effectiveTime < withoutTLB)

printf("TLB improves memory access time.\n");

else

printf("TLB does not improve memory access time.\n");

return 0;

}

Sample Output

Enter TLB lookup time (ns): 10

Enter Memory access time (ns): 100

Enter TLB hit ratio (0 to 1): 0.90

TLB PERFORMANCE

TLB Hit Time           : 110.00 ns

TLB Miss Time          : 210.00 ns

Effective Access Time : 120.00 ns

Without TLB            : 200.00 ns

TLB improves memory access time.

Calculation

TLB Hit Time = 10 + 100

= 110 ns

TLB Miss Time = 10 + 2(100)

= 210 ns

EAT = 0.90(110) + 0.10(210)

= 99 + 21

= 120 ns

Without TLB:

2 × 100 = 200 ns

Therefore:

With TLB    = 120 ns

Without TLB = 200 ns

The TLB reduces the effective memory access time.

Result

Thus, the program successfully simulates the Translation Lookaside Buffer and demonstrates that a high TLB hit ratio significantly improves memory access performance.

5. Memory Protection Using Base and Limit Registers

Aim

To implement a C program that demonstrates memory protection using base and limit registers and prevents a process from accessing memory outside its allocated range.

Concept

Base and limit registers provide hardware-supported memory protection.

- Base register → starting physical address of the process.

- Limit register → size of the memory region allocated to the process.

- Logical addresses must satisfy:

0 ≤ Logical Address < Limit

If the condition is satisfied:

Physical Address = Base + Logical Address

Otherwise, the operating system generates a protection violation.

Algorithm

- Start.

- Read base register.

- Read limit register.

- Read logical address.

- Check whether logical address is non-negative.

- Check whether logical address is less than the limit.

- If valid, calculate physical address.

- If invalid, deny memory access.

- Display the appropriate message.

- Stop.

C Program

#include <stdio.h>

int main()

{

int base, limit, logicalAddress;

int physicalAddress;

printf("----- MEMORY PROTECTION USING BASE AND LIMIT -----\n");

printf("Enter Base Register: ");

scanf("%d", &base);

printf("Enter Limit Register: ");

scanf("%d", &limit);

printf("Enter Logical Address: ");

scanf("%d", &logicalAddress);

if (logicalAddress >= 0 && logicalAddress < limit)

{

physicalAddress = base + logicalAddress;

printf("\nACCESS GRANTED\n");

printf("-------------------------\n");

printf("Logical Address  : %d\n", logicalAddress);

printf("Base Register    : %d\n", base);

printf("Limit Register   : %d\n", limit);

printf("Physical Address : %d\n", physicalAddress);

printf("Memory access is within the permitted range.\n");

}

else

{

printf("\nACCESS DENIED\n");

printf("-------------------------\n");

printf("Logical Address : %d\n", logicalAddress);

printf("Memory protection violation!\n");

printf("Address is outside the allocated memory range.\n");

}

return 0;

}

Sample Output – Authorized Access

MEMORY PROTECTION USING BASE AND LIMIT

Enter Base Register: 5000

Enter Limit Register: 1000

Enter Logical Address: 750

ACCESS GRANTED

Logical Address  : 750

Base Register    : 5000

Limit Register   : 1000

Physical Address : 5750

Memory access is within the permitted range.

Sample Output – Unauthorized Access

MEMORY PROTECTION USING BASE AND LIMIT

Enter Base Register: 5000

Enter Limit Register: 1000

Enter Logical Address: 1200

ACCESS DENIED

Logical Address : 1200

Memory protection violation!

Address is outside the allocated memory range.

Result

Thus, the program successfully demonstrates memory protection using base and limit registers by allowing only valid logical addresses and preventing unauthorized memory access.